# Nexum Suum Training Module

Thermodynamics & Boiler Maintenance – Modules 5 & 6

Prepared by Nexum Suum Incorporated – Efficiency | Ethics | Leadership | Compliance

# Module 5 – Compliance Standards

## 5.1 OSHA (Occupational Safety & Health Administration)

Focus: Operator safety, lockout/tagout (LOTO), confined space, and PPE.
Key Rule: Boilers are considered pressure vessels; improper access or untrained operation is a violation.
Why It Matters: Protects workers from burns, explosions, and electrocution.
Complexity: OSHA rules overlap with state boiler boards and must be cross-checked before audits.

## 5.2 ASME (American Society of Mechanical Engineers)

Boiler & Pressure Vessel Code (BPVC) defines design, construction, and testing.
• Section I – Power Boilers (high-pressure).
• Section IV – Heating Boilers (low-pressure).
Why It Matters: Compliance proves equipment integrity. Insurance carriers often demand ASME certification.

## 5.3 EPA (Environmental Protection Agency)

Focus: Emissions – NOx, SO2, CO, CO2, particulates.
Standards: Boilers must meet emission limits per fuel type.
Why It Matters: Noncompliance = heavy fines, reputational risk, environmental harm.
Example: An improperly tuned burner producing 400 ppm CO violates EPA air permits.

## 5.4 NFPA (National Fire Protection Association)

Codes: NFPA 85 (Boiler & Combustion Systems Hazards Code).
Focus: Fuel systems, flame safeguards, ignition interlocks.
Why It Matters: Ensures that explosions and fire hazards are mitigated.

## 5.5 Local & State Codes

Examples: NJ Boiler Law, NYC DOB Codes, California Air Resources Board.
Why It Matters: Local inspectors have authority to shut down equipment immediately if noncompliant.

# Module 6 – Common Faults, Issues & Solutions

## 6.1 Boiler-Side Problems

• Scale Buildup: Reduces conduction, raises fuel use. Solution: Water treatment, blowdown.
• Corrosion: Tube leaks and vessel thinning. Solution: Oxygen scavengers, DA checks.
• Low-Water Events: Explosion risk. Solution: Regular LWCO testing.
• Carryover (Wet Steam): Water droplets into steam lines. Solution: Proper drum level control, separators.

## 6.2 Combustion Faults

• Improper Air/Fuel Ratio: Too much air = loss, too little = CO/soot.
• Flame Instability: Burner trips and safety shutdowns.
Solution: Calibrated O₂/CO₂ analyzers, adjusted combustion controls.

## 6.3 Water-Side Issues

• Foaming/Priming: High TDS, poor steam quality.
• Oxygen Pitting: Poor DA performance.
Solution: Monitor water chemistry (phosphates, sulfites, pH control).

## 6.4 Controls & Interlocks

• Failed LWCO: Allows dry-firing.
• Faulty Flame Safeguard: Fails to detect ignition loss.
Solution: Weekly/monthly testing, recordkeeping.

## 6.5 Scenarios

Scenario 1 – Scale Buildup: Rising stack temp with no load increase. Diagnosis: tube fouling. Solution: acid clean.

Scenario 2 – Corrosion: High dissolved oxygen. DA vent not functioning. Solution: repair vent, add scavenger.

Scenario 3 – Interlock Failure: Boiler firing with empty gauge glass. LWCO failed. Action: shut down, test, replace LWCO.