# Nexum Suum Compliance Reference: Boilers, Chillers & Vital Facility Systems

## 1. Federal Compliance Framework

| Agency / Department | Code / Regulation | Primary Focus | Typical Penalties / Fines | Compliance Standards & Notes |
| --- | --- | --- | --- | --- |
| ASME | Section I, IV, VI, VII, VIII, IX, CSD-1 | Boiler & pressure vessel design | $25,000/day | Requires certification, inspection, and controls per CSD-1. |
| NBIC | NBIC Parts 1–3 | Inspection and repair of vessels | State fines | Annual inspection by authorized inspector required. |
| OSHA | 29 CFR 1910 Subparts L, Q, Z, 147 | Safety, LOTO, confined space | $15,625–$156,259 | PPE, energy isolation, confined space permits required. |
| EPA | 40 CFR Parts 60, 82, 403, 63 | Air, refrigerant, effluent, emissions | $50,000/day | Boiler tuning; refrigerant recovery; leak repair compliance. |
| DOE | 10 CFR 431 & 433 | Energy efficiency | Per-unit penalties | Boilers ≥80% efficiency; benchmarking compliance. |
| FDA | 21 CFR 110–117, 210–211 | Process steam & water purity | Product recall, consent decrees | Clean steam, condensate return, purity verification. |
| EHS | Varies by jurisdiction | Worker health, chemical storage | $5,000–$25,000 | SDS access, signage, eyewash, segregation required. |
| USDOL | 29 USC 651–678 | Training, records, operators | $15,000 per worker | Certified operators; LOTO & safety documentation. |
| ENERGY STAR / EPACT | EPACT 2005 / Energy Star | Efficiency & benchmarking | Program denial | Encourages >85% boiler efficiency. |
| ASHRAE / ANSI | 15, 34, 62.1, 90.1 | Refrigeration, IAQ, energy | Local enforcement | Defines refrigerant class & IAQ standards. |
| CSD-1 | ASME CSD-1 | Boiler controls & devices | Shutdown & fines | Annual testing of LWCO, flame safeguard, pressure controls. |

## 2. State and Local Enforcement Examples (NJ / Regional Model)

| Regulatory Body | Code / Statute | Applies To | Common Violations | Penalties / Fines |
| --- | --- | --- | --- | --- |
| NJ Department of Labor | N.J.A.C. 12:90 / N.J.S.A. 34:7-14 | Boilers >10 PSI / 10 HP | Expired inspection, unlicensed operator | $500–$2,000/day; red-tag shutdown |
| NJDEP – Air Quality | N.J.A.C. 7:27-19.16 | Boiler emissions | Missed annual tune-up | $10,000–$25,000/day |
| Municipal Fire Dept | NFPA 31/54/85 | Boilers & fuel systems | Leaking valves, missing reliefs | $5,000 + stop-work order |
| Public Health / Building | Local Codes | Cooling towers / water systems | Legionella testing failure | $2,000 per occurrence |
| County EHS | Clean Water Act | Blowdown & condensate | Unneutralized discharge | $25,000/day; system shutdown |

## 3. Compliance Standards by System Type

### A. Boiler Systems

Primary Standards: ASME I, IV, VI, VII, CSD-1; NBIC 1–3; OSHA 1910.147; EPA 40 CFR 60; NFPA 85
Key Actions:
- Annual inspection, safety testing, and combustion tuning
- Maintain logs for pressure, temperature, and efficiency
- Flame safeguard, LWCO, and relief valve checks annually
Penalties: $2,000–$10,000/day for uninspected boilers; $25,000/day for emissions exceedance.

### B. Chiller Systems

Primary Standards: ASHRAE 15, 34; EPA 608; OSHA 1910.134; UL 1995; NFPA 70
Key Actions:
- Maintain refrigerant logs and leak checks quarterly
- Verify ventilation ≥0.5 CFM/sq ft; test pressure cutouts
Penalties: $37,500–$50,000 per refrigerant release; $10,000/day for leak repair delays.

### C. Vital Facilities Equipment

Includes Compressors, Cooling Towers, Water Treatment, Pumps, Generators.
Standards: OSHA 1910.219, EPA 40 CFR 403, NFPA 37, ANSI Z223.1, ISO 50001
Key Actions:
- Maintain lubrication, pressure, and vibration logs
- Implement LOTO & Arc-Flash boundaries
- Maintain SDS and emergency plans
Penalties: $25,000/day for EPA discharge; $15,625 per OSHA violation; $5,000 for missing SDS.

### Environmental, Energy & Efficiency Considerations

| Area | Regulating Body / Standard | Requirement | Impact of Non-Compliance |
| --- | --- | --- | --- |
| Energy Benchmarking | DOE / EPA Energy Star | Annual EUI reporting | Loss of incentives, certification removal |
| GHG Reporting | EPA 40 CFR 98 | CO₂e inventory | $25,000/day fine |
| Water Discharge | EPA / Local POTW | pH 6.0–9.0, oil <100 mg/L | Violation notice or shutdown |
| Hazardous Waste | EPA 40 CFR 273 | Proper disposal of lamps/batteries | $10,000–$25,000 per incident |
| Workforce Training | OSHA / USDOL | LOTO & operator training | $5,000–$15,000 per untrained worker |

Summary Guidance:
✅ Maintain annual inspection & documentation logs
✅ Tune combustion & verify refrigerant recovery annually
✅ Maintain SDS & chemical labels per EHS
✅ Test all emergency safety interlocks quarterly
✅ Integrate dashboards for efficiency tracking.